ISABE 2022 Workshop

Tutorial Worksheet

Design point performance – 1-spool turbojet engine (Classroom Example)

Compressor exit temperature, T3 = _____________ K

Compressor work, CW = ___________ MW

Combustor heat input, Qin = ________ MW

Fuel flow, Wff = _________ kg/s

Turbine exit temperature, T7 = ___________ K

Turbine exit pressure, P7 = __________ bar

Nozzle exit pressure, P8 = _________ bar

Nozzle exit temperature (static), t8 = __________ K

Nozzle exit velocity, V8 = _________ m/s

Nozzle exit area, A = _________ m2

Thrust, FN = __________ N

Off-design performance – 1-spool turbojet (PCN =0.7)

Iteration 1

Guessed pressure ratio = ____________

From compressor map, corresponding NDMF = _____________

From compressor map, corresponding efficiency = ___________

Compressor exit temperature, T3 = ________ K

Guessed ΔT45 from combustor map = _________ K

Combustor exit temperature, T5 = ___________ K

Combustor heat input, Qin = ___________ MW

Combustor fuel flow, Wff = ___________ kg/s

Guessed turbine ΔH/T = ___________

Corresponding turbine NDMF * CN = __________

Corresponding turbine efficiency = __________

Turbine exit temperature, T7 = ___________ K

Turbine exit pressure, P7 = ___________ bar

Rotational compatibility = Yes/No

Mass flow compatibility = Yes/No

Work compatibility = Yes/No

Iteration 1

Guessed pressure ratio = ____________

From compressor map, corresponding NDMF = _____________

From compressor map, corresponding efficiency = ___________

Compressor exit temperature, T3 = ________ K

Guessed ΔT45 from combustor map = _________ K

Combustor exit temperature, T5 = ___________ K

Combustor heat input, Qin = ___________ MW

Combustor fuel flow, Wff = ___________ kg/s

Guessed turbine ΔH/T = ___________

Corresponding turbine NDMF * CN = __________

Corresponding turbine efficiency = __________

Rotational compatibility = Yes/No

Mass flow compatibility = Yes/No

Work compatibility = Yes/No

Performance calculation

Nozzle exit pressure, P8 = _________ bar

Nozzle exit temperature (static), t8 = __________ K

Nozzle exit velocity, V8 = _________ m/s

Thrust, FN = __________ N

Simulation Exercise 1

| SL | Operating condition | Compressor |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Pressure ratio | Exit temperature | Efficiency | Work |
| 1 | Design point |  |  |  |  |
|  | TET = 1141K |  |  |  |  |
| 2 | OP1 |  |  |  |  |
|  | TET = 900K |  |  |  |  |
| 3 | OP2 |  |  |  |  |
|  | TET = 1000K |  |  |  |  |
| 4 | OP3 |  |  |  |  |
|  | TET = 1200K |  |  |  |  |
| 5 | OP4 |  |  |  |  |
|  | TET = 1300K |  |  |  |  |

| SL | Operating condition | Turbine |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Pressure ratio | Exit temperature | Efficiency | Work |
| 1 | Design point |  |  |  |  |
|  | TET = 1141K |  |  |  |  |
| 2 | OP1 |  |  |  |  |
|  | TET = 900K |  |  |  |  |
| 3 | OP2 |  |  |  |  |
|  | TET = 1000K |  |  |  |  |
| 4 | OP3 |  |  |  |  |
|  | TET = 1200K |  |  |  |  |
| 5 | OP4 |  |  |  |  |
|  | TET = 1300K |  |  |  |  |

| SL | Operating condition | Performance |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Net thrust | SFC | Mass flow | Fuel flow |
| 1 | Design point |  |  |  |  |
|  | TET = 1141K |  |  |  |  |
| 2 | OP1 |  |  |  |  |
|  | TET = 900K |  |  |  |  |
| 3 | OP2 |  |  |  |  |
|  | TET = 1000K |  |  |  |  |
| 4 | OP3 |  |  |  |  |
|  | TET = 1200K |  |  |  |  |
| 5 | OP4 |  |  |  |  |
|  | TET = 1300K |  |  |  |  |

Simulation Exercise 2

| SL | Operating condition | Compressor |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Pressure ratio | Exit temperature | Efficiency | Work |
| 1 | Design point |  |  |  |  |
|  | 0, 0, 0 |  |  |  |  |
| 2 | OP1 |  |  |  |  |
|  | 0, -15, 0 |  |  |  |  |
| 3 | OP2 |  |  |  |  |
|  | 2000, -10, 0.5 |  |  |  |  |
| 4 | OP3 |  |  |  |  |
|  | 5000, 0, 0.6 |  |  |  |  |
| 5 | OP4 |  |  |  |  |
|  | 8000, 10, 0.7 |  |  |  |  |
| 6 | OP5 |  |  |  |  |
|  | 10000, 0, 0.8 |  |  |  |  |
| 7 | OP6 |  |  |  |  |
|  | 6000, 20, 0.8 |  |  |  |  |
| 8 | OP7 |  |  |  |  |
|  | 0, 0, 0.6 |  |  |  |  |

| SL | Operating condition | Turbine |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Pressure ratio | Exit temperature | Efficiency | Work |
| 1 | Design point |  |  |  |  |
|  | 0, 0, 0 |  |  |  |  |
| 2 | OP1 |  |  |  |  |
|  | 0, -15, 0 |  |  |  |  |
| 3 | OP2 |  |  |  |  |
|  | 2000, -10, 0.5 |  |  |  |  |
| 4 | OP3 |  |  |  |  |
|  | 5000, 0, 0.6 |  |  |  |  |
| 5 | OP4 |  |  |  |  |
|  | 8000, 10, 0.7 |  |  |  |  |
| 6 | OP5 |  |  |  |  |
|  | 10000, 0, 0.8 |  |  |  |  |
| 7 | OP6 |  |  |  |  |
|  | 6000, 20, 0.8 |  |  |  |  |
| 8 | OP7 |  |  |  |  |
|  | 0, 0, 0.6 |  |  |  |  |

| SL | Operating condition | Performance |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Net thrust | SFC | Mass flow | Fuel flow |
| 1 | Design point |  |  |  |  |
|  | 0, 0, 0 |  |  |  |  |
| 2 | OP1 |  |  |  |  |
|  | 0, -15, 0 |  |  |  |  |
| 3 | OP2 |  |  |  |  |
|  | 2000, -10, 0.5 |  |  |  |  |
| 4 | OP3 |  |  |  |  |
|  | 5000, 0, 0.6 |  |  |  |  |
| 5 | OP4 |  |  |  |  |
|  | 8000, 10, 0.7 |  |  |  |  |
| 6 | OP5 |  |  |  |  |
|  | 10000, 0, 0.8 |  |  |  |  |
| 7 | OP6 |  |  |  |  |
|  | 6000, 20, 0.8 |  |  |  |  |
| 8 | OP7 |  |  |  |  |
|  | 0, 0, 0.6 |  |  |  |  |

Simulation Exercise 3

| Operating condition | Compressor faults |  |  | Turbine faults |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | PR Deg | FC Deg | ETA Deg | DH deg | TF deg | ETA deg |
| OP1 | 0% | -1% | -2% | 0% | 0% | 0% |
| OP2 | -2% | 0% | 0% | 0% | 0% | -2% |
| OP3 | 0% | 0% | 0% | -1% | 0% | -3% |
| OP4 | -1% | -1% | -3% | 0% | 0% | 0% |
| OP5 | 0% | 0% | -4% | 0% | 0% | -2% |
| OP6 | 0% | 0% | 0% | -2% | -3% | 0% |
| OP7 | -3% | 2% | -3% | 0% | -1% | -1% |
| OP8 | 0% | 1% | 0% | 0% | -2% | -3% |

| SL | Operating condition | Compressor |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Pressure ratio | Exit temperature | Efficiency | Work |
| 1 | Design point |  |  |  |  |
| 2 | OP1 |  |  |  |  |  |
| 3 | OP2 |  |  |  |  |  |
| 4 | OP3 |  |  |  |  |  |
| 5 | OP4 |  |  |  |  |  |
| 6 | OP5 |  |  |  |  |  |
| 7 | OP6 |  |  |  |  |  |
| 8 | OP7 |  |  |  |  |  |
| 9 | OP8 |  |  |  |  |  |

| SL | Operating condition | Turbine |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Pressure ratio | Exit temperature | Efficiency | Work |
| 1 | Design point |  |  |  |  |
| 2 | OP1 |  |  |  |  |  |
| 3 | OP2 |  |  |  |  |  |
| 4 | OP3 |  |  |  |  |  |
| 5 | OP4 |  |  |  |  |  |
| 6 | OP5 |  |  |  |  |  |
| 7 | OP6 |  |  |  |  |  |
| 8 | OP7 |  |  |  |  |  |
| 9 | OP8 |  |  |  |  |  |

| SL | Operating condition | Power Turbine |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Pressure ratio | Exit temperature | Efficiency | Work |
| 1 | Design point |  |  |  |  |
| 2 | OP1 |  |  |  |  |  |
| 3 | OP2 |  |  |  |  |  |
| 4 | OP3 |  |  |  |  |  |
| 5 | OP4 |  |  |  |  |  |
| 6 | OP5 |  |  |  |  |  |
| 7 | OP6 |  |  |  |  |  |
| 8 | OP7 |  |  |  |  |  |
| 9 | OP8 |  |  |  |  |  |

| SL | Operating condition | Performance |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Shaft Power | SFC | Mass flow | Fuel flow |
| 1 | Design point |  |  |  |  |
| 2 | OP1 |  |  |  |  |  |
| 3 | OP2 |  |  |  |  |  |
| 4 | OP3 |  |  |  |  |  |
| 5 | OP4 |  |  |  |  |  |
| 6 | OP5 |  |  |  |  |  |
| 7 | OP6 |  |  |  |  |  |
| 8 | OP7 |  |  |  |  |  |
| 9 | OP8 |  |  |  |  |  |

Simulation Exercise 4

| Operating condition | Time (mins) | Input |  |  |
| --- | --- | --- | --- | --- |
|  |  | Altitude (m) | Mach number | PCN |
| Taxi | 5 | 0 | 0 | 0.7 |
| Takeoff | 1 | 0 | 0 | 1.1 |
| Climb 1 | 5 | 3000 | 0.5 | 0.9 |
| Climb 2 | 10 | 7000 | 0.7 | 0.8 |
| Cruise | 123 | 10000 | 0.8 | 1 |
| Low Rating | 30 | 5000 | 0.75 | 0.7 |
| Reverse Thrust | 1 | 0 | 0.4 | 1.1 |
| Taxi | 5 | 0 | 0 | 0.7 |

Assuming that we have been able to determine the operating temperatures and
stress levels, from the Larson-Miller chart we can provide the parameter P in
the equation:

P= T1000(Log tf+20)

Where,
T = Temperature K (TET)
tf = Time to failure

The TET-Stress correlation is provided below:

| TET (K) | Stress (MPa) |
| --- | --- |
| 1570 | 350 |
| 1525 | 300 |
| 1290 | 265 |
| 1215 | 250 |
| 1070 | 200 |
| 1040 | 150 |
| 900 | 100 |

The stress-LMP correlation is provided below:

| Stress (MPa) | P |
| --- | --- |
| 100 | 37.0 |
| 150 | 36.0 |
| 200 | 35.0 |
| 250 | 34.5 |
| 265 | 34.1 |
| 300 | 34.0 |
| 350 | 33.9 |

| Operating condition | Time (mins) | Input |  |  | Output |  |  | Stress (Mpa) | P | tf (hours) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Altitude (m) | Mach number | PCN | Net Thrust | Fuel flow | Turbine inlet T11 |  |  |  |
| Taxi | 5 | 0 | 0 | 0.7 |  |  |  |  |  |  |
| Takeoff | 1 | 0 | 0 | 1.1 |  |  |  |  |  |  |
| Climb 1 | 5 | 3000 | 0.5 | 0.9 |  |  |  |  |  |  |
| Climb 2 | 10 | 7000 | 0.7 | 0.8 |  |  |  |  |  |  |
| Cruise | 123 | 10000 | 0.8 | 1 |  |  |  |  |  |  |
| Low Rating | 30 | 5000 | 0.75 | 0.7 |  |  |  |  |  |  |
| Reverse Thrust | 1 | 0 | 0.4 | 1.1 |  |  |  |  |  |  |
| Taxi | 5 | 0 | 0 | 0.7 |  |  |  |  |  |  |

| Operating condition | Time (mins) | tf (hours) | T/tf |
| --- | --- | --- | --- |
| Taxi | 5 |  |  |
| Takeoff | 1 |  |  |
| Climb 1 | 5 |  |  |
| Climb 2 | 10 |  |  |
| Cruise | 123 |  |  |
| Low Rating | 30 |  |  |
| Reverse Thrust | 1 |  |  |
| Taxi | 5 |  |  |
| Total | 180 |  |  |

- Total number of cycles using Miner’s Law, N = 1/(Total T/tf)

= ____________

- Applying FOS = 1.5, the number of cycles, Nfos = N/1.5

= ____________

- Total number of hours before failure criteria, T = Nfos x 180/60

= _____________

Simulation exercise 5

Using -3 in compressor efficiency degradation index (%) to represent fan degradation

| Operating condition | Time (mins) | Input |  |  | Output |  |  | Stress (Mpa) | P | tf (hours) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Altitude (m) | Mach number | PCN | Thrust | Fuel flow | Turbine inlet T11 |  |  |  |
| Taxi | 5 | 0 | 0 | 0.7 |  |  |  |  |  |  |
| Takeoff | 1 | 0 | 0 | 1.1 |  |  |  |  |  |  |
| Climb 1 | 5 | 3000 | 0.5 | 0.9 |  |  |  |  |  |  |
| Climb 2 | 10 | 7000 | 0.7 | 0.8 |  |  |  |  |  |  |
| Cruise | 123 | 10000 | 0.8 | 1 |  |  |  |  |  |  |
| Low Rating | 30 | 5000 | 0.75 | 0.7 |  |  |  |  |  |  |
| Reverse Thrust | 1 | 0 | 0.4 | 1.1 |  |  |  |  |  |  |
| Taxi | 5 | 0 | 0 | 0.7 |  |  |  |  |  |  |

| Operating condition | Time (mins) | tf (hours) | T/tf |
| --- | --- | --- | --- |
| Taxi | 5 |  |  |
| Takeoff | 1 |  |  |
| Climb 1 | 5 |  |  |
| Climb 2 | 10 |  |  |
| Cruise | 123 |  |  |
| Low Rating | 30 |  |  |
| Reverse Thrust | 1 |  |  |
| Taxi | 5 |  |  |
| Total | 180 |  |  |

- Total number of cycles using Miner’s Law, N = 1/(Total T/tf)

= ____________

- Applying FOS = 1.5, the number of cycles, Nfos = N/1.5

= ____________

- Total number of hours before failure criteria, T = Nfos x 180/60

= _____________